Quality Assurance Checklist (APEX X2s Only)

Save this file as a PDF using the joint name, e.g F-1234-5678-A-01-R432

- Joint Closure Details

| Contractor Company Name |  |
| --- | --- |
| Lead Engineer Name |  |
| Date |  |
| Location (What3Words) |  |

- Cable Installation

| CAU Configuration | Loose tube section of the CAU has been removed | Select | Take an Image of each Main Cable | Take an Image of each Spur Cable if installed |
| --- | --- | --- | --- | --- |
|  | Cable attachment unit is fixed to the cable with white spacer and spur bracket | Select |  |  |
|  | The CAU has been fully secured to the APEX Base | Select |  |  |
|  | The outer sheath cut is located above the gel seal | Select |  |  |
| Cable Setup | 80mm of Inner Sheath remains | Select |  |  |
|  | R-DISAF only: 20mm of the white shock absorber remains | Select |  |  |
|  | 220mm of water blocking tape loosely secured | Select |  |  |

- External Checks

| Crown Bracket | The crown bracket is installed | Select | Take an Image of the Crown Bracket |
| --- | --- | --- | --- |
|  | All Cables secured to the Crown Bracket using hose clamp with spur bracket at the top | Select |  |
| Blank Ports | Each unused cable port has blank plug installed | Select | Take an Image of the Blank Ports |
| Wedge Seal | Each cable port wedge seal screw is fully tightened | Select | Take an Image of the Wedge Seals |

- Fibre Routing

| Basket | The water blocking tape section of the cable is secured to the basket | Select | Take an Image of the Slack Basket |
| --- | --- | --- | --- |
|  | The loop of fibre in the basket is secured to the basket using Velcro | Select |  |
| Spiral Wrap | 320mm of spiral wrap has been applied to each unit bundle | Select | Take an Image of the Spiral Wrap |
|  | The Spiral wrapped fibre passes behind the splice tray spine | Select |  |
|  | All the Spiral wraps are secured to the basket using Velcro | Select |  |

- Splice Tray

| Splice Tray | The spiral wrapped fibre is secured to the splice tray using foam and cable ties | Select | Take an Image of each Splice Tray (Blue at the top, white at the bottom |
| --- | --- | --- | --- |
|  | The unit bundle binder tape has been tied off at the end of the spiral wrap | Select |  |
|  | A minimum of 3 loops of slack stored in the splice tray >1m | Select |  |
|  | Each splice tray lid has been installed | Select |  |
|  | Correct Unit bundle order. Tray 1 (Top): Blue Tray 6 (Bottom): White | Select |  |
| Splice Tray Identifier Labels | The cable identifier label has been attached to the end of the splice trays | Select | Take an end Image of the Splice Trays |

- APEX Seal

| O-Ring | The APEX O-Ring is attached and clean | Select | Take an Image of the O-Ring |
| --- | --- | --- | --- |
| Clamp ring | The clamp ring has been fully locked | Select | Take an Image of the clamp ring closed |

- Joint Chamber

| Critchley label | Critchley label highlighting the main joint name and or spur name (F-1234-5678-A-01-R432) (F-SPUR-ABCD-A-R48 | Select | Take an Image of the Label |
| --- | --- | --- | --- |
| STAKKA box or equivalent Location ID Tagged | Storage of the slack is neatly stored with the APEX closure sat on the chamber arms Include ELR/Mileage | Select | Take an Image of the chamber with lids off |
| STAKKA box or equivalent | The lids of the chamber are on, fitted correctly and secured | Select | Take an Image of the chamber with lids on |